	
	ASC OFF SWITCH


	

	This switch is mounted on the lower panel, and turns off and on the system.

SYSTEM CONTROL STATUS BY ASC OFF SWITCH
ASC OFF switch operation mode
Transfer position and rear differential lock status
Active stability control system
Active traction control system
ABS (EBD)
Brake control
Engine control
Deactivated

2H

Control activated

Control activated

Control activated

Control activated

4H

Control activated

Control activated

Control activated

Control activated

4HLc

Control activated

Control activated

Control deactivated

Control activated

4LLc

Control deactivated

Control activated

Control deactivated

Control activated

4HLc/Rear differential lock

Control deactivated

Control deactivated

Control deactivated

Control deactivated

4LLc/Rear differential lock

Control deactivated

Control deactivated

Control deactivated

Control deactivated

Activated

2H

Control deactivated

Control activated

Control deactivated

Control activated

4H

Control deactivated

Control activated

Control deactivated

Control activated

4HLc

Control deactivated

Control activated

Control deactivated

Control activated

4LLc

Control deactivated

Control activated

Control deactivated

Control activated

4HLc/Rear differential lock

Control deactivated

Control deactivated

Control deactivated

Control deactivated

4LLc/Rear differential lock

Control deactivated

Control deactivated

Control deactivated

Control deactivated

note
· If Active Stability Control system is deactivated (OFF) by the ASC OFF switch, activation (ON) will be automatically resumed when the vehicle speed becomes 120 km/h or more.

· Active Stability Control system can be deactivated (OFF) when the vehicle speed becomes 40 km/h or less.




