[edit] 4D56
Displacement - 2.5 L (2,476 cc)
Bore - 91.1 mm
Stroke - 95.0 mm
Fuel type - Diesel

[edit] Non-Turbo
· Power - 51 kW (68 hp) @ 4200 rpm 

· Torque - 142 N·m (105 ft·lb) @ 2500 rpm 

· Engine type - Inline 4 cylinder SOHC 

· Fuel system - Distribution type jet pump 

· Compression ratio - 21:1 

[edit] Non-intercooled Turbo (TD04 Turbo)              
· Power - 67 kW (90 hp) @ 4200 rpm                     
· Torque - 197 N·m (145 ft·lb) @ 2000 rpm 

· Engine type - Inline 4 cylinder SOHC 

· Fuel system - Distribution type jet pump                     
· Compression ratio - 21:1[10] 

[edit] Intercooled Turbo (TD04 watercooled Turbo)
· Power - 78 kW (104 hp) @ 4200 rpm 

· Torque - 240 N·m (177 ft·lb) @ 2000 rpm                          
· Engine type - Inline 4 cylinder SOHC 

· Fuel system - Distribution type jet pump 

· Compression ratio - 21:1 

[edit] Intercooled Turbo (1st Generation DI-D)
· Power - 85 kW (114 hp) @ 4000 rpm 

· Torque - 247 N·m (182 ft·lb) @ 2000 rpm 

· Engine type - Inline 4 cylinder 

· Fuel system - 1st Generation Common Rail Direct Injection (CRDi) 

· Compression ratio - 17:1[11]

 HYPERLINK "http://en.wikipedia.org/wiki/Mitsubishi_Astron_engine" \l "cite_note-11#cite_note-11" [12]

 HYPERLINK "http://en.wikipedia.org/wiki/Mitsubishi_Astron_engine" \l "cite_note-12#cite_note-12" [13] 

[edit] Intercooled Turbo (2nd Generation DI-D)
· Power - 103 kW (138 hp) @ 4000 rpm 

· Torque - 320 N·m (236 ft·lb) @ 2000 rpm 

· Engine type - Inline 4 cylinder 

· Fuel system - 2nd Generation Common Rail Direct Injection (CRDi) 

· Compression ratio - 17:1[14]

 HYPERLINK "http://en.wikipedia.org/wiki/Mitsubishi_Astron_engine" \l "cite_note-14#cite_note-14" [15]

 HYPERLINK "http://en.wikipedia.org/wiki/Mitsubishi_Astron_engine" \l "cite_note-15#cite_note-15" [16]

 HYPERLINK "http://en.wikipedia.org/wiki/Mitsubishi_Astron_engine" \l "cite_note-16#cite_note-16" [17] 

[edit] Intercooled Turbo (3rd Generation DI-D with variable geometry turbo)
· Power - 130 kW (175 hp) @ 4000 rpm 

· Torque - 400 N·m (295 ft·lb) @ 2000 rpm 

· Engine type - Inline 4 cylinder 

· Fuel system - 2nd Generation Common Rail Direct Injection (CRDi) 

· Compression ratio - 16.5:1[18] 

