	Item No.
	Check item
	Inspection conditions
	Value to be determined as normal
	Code No. or inspection procedure No.
	Reference page

	16
	Exhaust gas recirculation valve position sensor
	· Engine: After warming up the engine

· Transmission: Neutral <M/T> or P range <A/T>


	Idling
	50% or more*
	Code No. P0405, P0406
	

, 



	
	
	
	3,500 r/min or more
	10% or below
	
	

	17
	Exhaust gas recirculation valve target position
	· Engine: After warming up the engine

· Transmission: Neutral <M/T> or P range <A/T>


	Idling
	50% or more
	Code No. P0405, P0406
	

, 



	
	
	
	
	
	
	
	

	note
	*: At an outside temperature of approximately 20°C. When the exhaust gas recirculation valve position sensor is checked at the outside temperature of 20°C or more, the value would be possibly below the opening degree (normal value). Also, the value would be possibly below the opening degree (normal value) under the following conditions:


	Terminal No.
	Check item
	Inspection condition (engine status)
	Normal condition

	19
	Exhaust gas recirculation motor (+)
	Engine: Idling (after warning up)
	1.8 V or higher*

	
	
	Engine: 3,500 r/min (after warning up)
	1.6 V or lower*

	20
	Exhaust gas recirculation motor (-)
	Engine: Stopped from idling speed
	1 V or lower → 2 V
Changes repeatedly* (After several seconds have elapsed)

	51
	Exhaust gas recirculation position sensor
	Engine: Idling (after warning up)
	2.0 V or higher

	
	
	Engine: 3,500 r/min (after warning up)
	1.5 V or lower


