| Item |  | Standard value |
| --- | --- | --- |
| Resistance between glow plug plate and glow plug body (parallel resistance for 4 glow plugs) (at 20°C) Ω |  | 0.12 - 0.38 |
| Voltage between glow plug plate and glow plug body V | Immediately after ignition switch is turned to ON (without starting the engine) | 9 - 11 (Drops to 0 V after 4 - 8 seconds have passed) |
|  | While engine is cranking | 6 or more |
|  | While engine is warning up | 12 - 15 (Drops to 0 V when the engine coolant temperature increases to 60°C or more or if 180 seconds have passed since the engine was started) |
| Glow plug relay resistance (at 20°C) Ω |  | 18 - 22 |
| Glow plug resistance (at 20°C) Ω |  | 0.5 - 1.5 |