|  | 1.Check that the battery voltage is 11 - 13 V. |
| --- | --- |

|  | 2.Check that the engine coolant temperature is 40°C or less. note; If the engine coolant temperature is too high, disconnect the engine coolant temperature sensor connector.. |
| --- | --- |

|  | 3.Measure the resistance between the glow plug plate and the glow plug body (earth). Standard value: 0.12 - 0.38 Ω (at 20°C) note; The resistance value is the parallel resistance value for the four glow plugs.. 4.Connect a voltmeter between the glow plug plate and the glow plug body (earth). 5.Measure the voltage immediately after the ignition switch is turned to ON (without starting the engine). Standard value: 9 - 11 V (Drops to 0 V after 4 - 8 seconds have passed) In addition, check to be sure that the glow indicator lamp (red) illuminates immediately after the ignition switch is turned to ON. note; The time during which the voltage appears (energizing time) will depend on the engine coolant temperature.. 6.Measure the voltage while the engine is cranking. Standard value: 6 V or more 7.Start the engine and measure the voltage while the engine is warming up. However, if the engine coolant temperature rises above 60°C or when 180 seconds have passed since the engine was started, the voltage will always return to 0 V. (Refer to the Glow Plug Energization Timing Chart.) Standard value: 12 - 15 V |
| --- | --- |

|  | Glow Plug Energization Timing Chart <Reference> |
| --- | --- |